HYL-030DA0700G097AD_DALI 2

Constant current LED driver

DALI Dimmable

Product description

- Dimmable Independent constant current LED Driver

- Adjustable output current between350 and 700mA

via DIP switch, Factory default setting 350mA gear

- Max. output power 30W

- Up to 85 % efficiency

- Power input on stand-by < 0.3 W

- Dimming range 1 – 100 %

- For luminaires of protection class I and protection class II

- Nominal life-time up to 50,000 h

- 5-year guarantee

Benefits

- Application-oriented operating window for maximum compatibility

- Best energy savings due to low stand-by losses and high efficiency

- Long lasting and high reliability

- Classic compact housing

Interfaces

- DALI （DT-6）

- SwitchDIM\PUSH DIM (with memory function)

- Terminal blocks: 45° push terminals

Applications

- Linear and area lighting

- Office – industrial – shop

Approval marks

In preparation, if not already printed on product label

Technical data

Units：mm

ON

OFF

●

○

DIP Switch

DIP Switch

ON

OFF

Ordering data

Specific technical data

①Load switching on output side is safe but not permitted

②Valid at 100 % dimming level

| ACCESSORIES |  |
| --- | --- |

Product description

- Optional strain-relief set for independent applications

- Transforms the LED Driver into a fully class II compatible LED Driver

(e.g. ceiling installation)

Figure 2

Figure 1

Ordering data